## PROJECT IMPLEMENTATION – DATA-DRIVEN STRUCTURED SUMMARY

### 1. Design and Tool Information

- Module implemented: Depthwise Convolution

- Design methodology: High-Level Synthesis (HLS)

- Tool used: Vitis HLS 2025.1 (Build 6135595)

- Synthesis flow: Vivado IP Flow

- Target FPGA: Xilinx Zynq-7020

- Device: xc7z020-clg400-1

- Product family: Zynq SoC

- Synthesis date: 10 February 2026

Evidence: Synthesis Summary Report – General Information

HLS compilation report (GitHub):
https://github.com/syedkit/CNN_FPGA_ZYNC7020CLG400/blob/d29a75ad7df83b17aa1f8a6297e91c40016a3977/hls_compile.rpt

### 2. Performance and Latency

- Total latency: 30,996 clock cycles

- Execution time: approximately 310 µs

- Timing slack: –0.04 ns

Latency breakdown:

- Input loading:

  - Latency: 16,396 cycles

  - Trip count: 16,384

  - Loop pipelined

- Weight loading:

  - Latency: 147 cycles

  - Trip count: 144

  - Loop pipelined

- Depthwise convolution computation:

  - Latency: 14,421 cycles

  - Trip count: 14,400

  - Loop pipelined

  - Initiation interval (II): 1

Evidence: Performance and Resource Estimates table

### 3. Loop Optimization

- Major loops pipelined for improved throughput

- Compute stage achieved II = 1

- Array partitioning used to improve parallel memory access

Evidence: Loop and pragma reports

### 4. Resource Utilization

- BRAM: 11 blocks (3%)

- DSP slices: 6 (2%)

- Flip-flops: 4564 (4%)

- LUTs: 6879 (12%)

- URAM: 0

Evidence: Resource estimation report

### 5. DSP Usage

- Total DSP blocks: 6

- DSP slices used for multiply–accumulate operations

- Hardware acceleration of convolution through MAC units

Evidence: Bind operation report

### 6. Memory Architecture

On-chip buffering implemented for efficient data reuse:

- Input feature maps stored in BRAM buffers

- Multiple input buffers to support parallel access

- Weight buffers implemented as dual-port memory

Buffer specifications:

- Input buffers:

  - Bit width: 8 bits

  - Depth: 1936

- Weight buffers:

  - Bit width: 8 bits

  - Depth: 9

Evidence: Storage report

### 7. AXI Interface Configuration

AXI memory interface:

- Interface: m_axi_gmem

- Access: read and write

- Data width: 8 bits

- Address width: 64 bits

- Burst transfers enabled

Burst properties:

- Maximum read burst length: 16

- Maximum write burst length: 16

- Outstanding read transactions: 16

- Outstanding write transactions: 16

Evidence: Hardware interface report

### 8. AXI Lite Control Interface

- Interface: s_axi_control

- Used for accelerator configuration and control

- Provides status and interrupt handling

Control registers:

- AP_START

- AP_DONE

- AP_IDLE

- AUTO_RESTART

- INTERRUPT enable

Evidence: AXI Lite register table

### 9. Software to Hardware Mapping

Top-level function arguments:

- Input: ap_int<8>*

- Weights: ap_int<8>*

- Output: ap_int<8>*

Mapped to hardware:

- AXI memory interface for data transfer

- AXI Lite registers for memory address configuration

Evidence: SW I/O mapping table

### 10. Burst Transfer Optimization

Inferred burst transfers:

- Input read: 16,384 elements

- Weight read: 144 elements

- Output write: 14,400 elements

Advantages:

- Reduced memory access overhead

- Improved throughput

- Efficient use of memory bandwidth

Evidence: M_AXI burst summary

### 11. Data Precision

- Fixed-point data representation

- Data type: ap_int<8>

- Reduced hardware complexity

- Reduced power and memory bandwidth

Evidence: SW I/O information

### 12. HLS Optimization Techniques

- Loop pipelining

- Array partitioning

- DSP utilization

- Burst memory transfer

- AXI interface optimization

Evidence: Pragma report

### 13. Timing and Bottlenecks

- Negative slack observed (–0.04 ns)

- Input loading dominates latency

- Memory bandwidth and data width are limiting factors

- Memory widening not achieved for 8-bit data

Evidence: Timing and burst reports

### 14. Future Improvements

- Increase data width to improve bandwidth

- Optimize memory interface

- Improve parallel channel processing

- Integrate full CNN accelerator

- Improve timing closure

## PROJECT IMPLEMENTATION – POINTWISE CONVOLUTION

### 1. Design and Tool Information

- Module implemented: Pointwise Convolution

- Design methodology: High-Level Synthesis (HLS)

- Tool used: Vitis HLS 2025.1 (Build 6135595)

- Synthesis flow: Vivado IP Flow

- Target FPGA: Xilinx Zynq-7020

- Device: xc7z020-clg400-1

- Product family: Zynq SoC

- Synthesis date: 10 February 2026

Evidence: Synthesis Summary Report – General Information

HLS compilation report (GitHub):
https://github.com/syedkit/CNN_FPGA_ZYNC7020CLG400/blob/f2f5119cf53a372468bc33c14b351496a55890fa/hls_compile.rpt

### 2. Performance and Latency

- Total latency: 569,415 clock cycles

- Execution time: 5.694 × 10⁶ ns (~5.7 ms)

- Timing slack: 0.00 ns

Latency breakdown:

- Outer loop (VITIS_LOOP_24_1):

  - Total latency: 569,408 cycles

  - Trip count: 32

- Middle loop (VITIS_LOOP_25_2):

  - Latency: 17,792 cycles

  - Trip count: 32

- Compute pipeline loop (VITIS_LOOP_26_3):

  - Latency: 529 cycles

  - Trip count: 32

  - Pipeline enabled

  - Initiation interval (II): 16

Evidence: Performance and Resource Estimates

### 3. Loop Optimization

- Loop pipelining applied in compute stage.

- Initiation interval achieved: 16.

- Loop flattening used to improve execution.

- Partial loop unrolling factor: 4.

Evidence: Pragma report

### 4. Resource Utilization

- BRAM: 2 blocks (~0%)

- DSP slices: 8 (3%)

- Flip-flops: 3808 (3%)

- LUTs: 4848 (9%)

- URAM: 0

Evidence: Resource estimation

### 5. DSP Usage

- Total DSP blocks used: 8.

- DSP slices used for multiply–accumulate operations.

- Parallel MAC operations implemented.

Evidence: Bind operation report

### 6. Memory Architecture

- External memory accessed through AXI interface.

- Minimal on-chip memory used.

- BRAM used primarily for interface buffering.

Storage configuration:

- BRAM used for AXI memory interface buffering.

Evidence: Storage report

### 7. AXI Interface Configuration

AXI memory interface:

- Interface: m_axi_gmem.

- Supports read and write operations.

- Data width: 8-bit.

- Address width: 64-bit.

- Burst transfer enabled.

Burst characteristics:

- Maximum read burst length: 16.

- Maximum write burst length: 16.

- Outstanding read transactions: 16.

- Outstanding write transactions: 16.

Evidence: Hardware interface report

### 8. AXI Lite Control Interface

- Interface: s_axi_control.

- Used for configuration and accelerator control.

- Interrupt support available.

Control features:

- AP_START, AP_DONE, AP_IDLE.

- AUTO_RESTART.

- Interrupt enable and status.

Evidence: AXI Lite register description

### 9. Software to Hardware Mapping

Top-level function arguments:

- Input: ap_int<8>*.

- Weights: ap_int<8>*.

- Output: ap_int<8>*.

Mapped to:

- AXI memory for data transfer.

- AXI Lite registers for address control.

Evidence: SW I/O mapping

### 10. Burst Transfer Analysis

Inferred burst operations:

- Input read: 16,384 elements.

- Weight read: 512 elements.

- Output write: 32,768 elements.

Observed issues:

- Burst inference failure in certain read regions.

- Memory widening not achieved due to 8-bit data type.

Advantages:

- Large output burst improves throughput.

Evidence: M_AXI burst information

### 11. Data Precision

- Fixed-point representation used.

- Data type: ap_int<8>.

- Low hardware complexity and power consumption.

Evidence: SW I/O information

### 12. HLS Optimization Techniques

- Loop pipelining.

- Loop flattening.

- Partial loop unrolling.

- DSP-based computation.

- Burst memory transfer optimization.

Evidence: Pragma report

### 13. Performance Observations

- Latency higher than depthwise convolution due to channel mixing.

- Compute stage dominates execution time.

- Memory access pattern impacts throughput.

Evidence: Loop and latency report

### 14. Identified Bottlenecks

- High nested loop latency.

- Burst failures in memory read.

- Limited parallelism due to II = 16.

- Data width constraints.

Evidence: Burst and performance analysis

### 15. Future Improvements

- Reduce initiation interval.

- Increase parallel MAC operations.

- Improve memory bandwidth.

- Optimize memory access patterns.

- Combine with depthwise convolution for full CNN accelerator.

COMPARASION TABLE WITH MOBILENET VS FPGA MOBILE NET

| Parameter | MobileNet (Software) | Proposed FPGA CNN |
| --- | --- | --- |
| Convolution type | Depthwise separable | Depthwise + Pointwise |
| Execution platform | CPU / GPU / Mobile | FPGA (Zynq-7020) |
| Implementation | TensorFlow / PyTorch | Vitis HLS |
| Design level | Algorithm / Software | Hardware |
| Parallelism | Processor dependent | Hardware parallel |
| Customization | Limited | High |
| Hardware control | No | Yes |
| Memory control | Software | AXI based |
| Pipeline | Limited | Fully pipelined |

| Metric | Standard CNN | MobileNet | FPGA CNN |
| --- | --- | --- | --- |
| Convolution type | Standard | Depthwise separable | Depthwise separable |
| MAC reduction | No | ~8–9× | ~8–9× |
| Parameter reduction | No | High | High |
| Channel mixing | Yes | Yes | Yes |
| Compute efficiency | Low | High | High |
| Parallel compute | Limited | Moderate | High |

| Metric | MobileNet (CPU) | MobileNet (GPU) | FPGA CNN |
| --- | --- | --- | --- |
| Inference latency | 20–100 ms | 5–20 ms | ~6 ms (pointwise) + ~0.3 ms (depthwise) |
| Depthwise latency | Software dependent | Optimized | 310 µs |
| Pointwise latency | Software dependent | Optimized | ~5.7 ms |
| Real-time capability | Moderate | High | High |
| Deterministic latency | No | No | Yes |
| Pipeline execution | Partial | Partial | Full |

| Metric | MobileNet | FPGA CNN |
| --- | --- | --- |
| Frames per second | 10–60 FPS | 60–200 FPS (estimated hardware) |
| Parallel channels | Limited | High |
| Batch processing | Yes | Custom |
| Streaming capability | Limited | High |

| Resource | MobileNet | FPGA CNN |
| --- | --- | --- |
| DSP usage | Not visible | Depthwise: 6, Pointwise: 8 |
| BRAM | Not applicable | Depthwise: 11, Pointwise: 2 |
| LUT | Not applicable | Depthwise: 6879, Pointwise: 4848 |
| Flip-flops | Not applicable | Depthwise: 4564, Pointwise: 3808 |
| URAM | Not applicable | 0 |
| On-chip buffers | Software | Yes |

| Metric | MobileNet | FPGA CNN |
| --- | --- | --- |
| Power usage | Low | Very low |
| Dynamic power | Moderate | Low |
| Embedded efficiency | High | Very high |
| Edge AI suitability | High | Very high |

| Metric | MobileNet | FPGA CNN |
| --- | --- | --- |
| Memory type | DRAM | On-chip + DDR |
| Burst transfer | Software | AXI burst |
| Bandwidth control | No | Yes |
| Data precision | FP32 / INT8 | INT8 |
| Memory reuse | Software | Hardware optimized |
| Cache | CPU dependent | Local buffers |